| BIT BYTE | 7 | 6 | 5 | 4 |  | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Operation Code (D8h) |  |  |  |  |  |  |  |  |
| 1 (1*) | Read(0x1) / Write(0x0) |  |  |  | NORMAL( 0x5 ) / PREFIX( 0x0 ) |  |  |  |  |
| 2 (2*) | Reserved |  |  |  |  |  |  |  |  |
| 3 | Feature Register ( 0:7 ) |  |  |  |  |  |  |  |  |
| 4 (3*) | Check Word (0x06) |  |  |  |  |  |  |  |  |
| 5 (3*) | Check Word (0x7B) |  |  |  |  |  |  |  |  |
| 6 (4*) | Length (MSB) |  |  |  |  |  |  |  |  |
| 7 (4*) | Length |  |  |  |  |  |  |  |  |
| 8 (4*) | Length |  |  |  |  |  |  |  |  |
| 9 (4*) | Length (LSB) |  |  |  |  |  |  |  |  |
| 10 | Sector Count Register ( 0:7 ) |  |  |  |  |  |  |  |  |
| 11 | LBA Low Register ( 0:7 ) |  |  |  |  |  |  |  |  |
| 12 | LBA Mid Register ( 0:7 ) |  |  |  |  |  |  |  |  |
| 13 | LBA High Register ( 0:7 ) |  |  |  |  |  |  |  |  |
| 14 (5*) | Device/Head Register |  |  |  |  |  |  |  |  |
| 15 (6*) | Command Register (only support PIO commands) |  |  |  |  |  |  |  |  |

(1*) For Example, 0x15 indicates Data In.

(2*) Don’t care

(3*) Vendor ID magic

(4*) Length in bytes, this is essential.

(5*) The value is depended on command.

(6*) D8 command is only supported to use PIO command.

Others fields, please refer to SATA spec.

| BIT BYTE | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Status |  |  |  |  |  |  |  |
| 1 | Error |  |  |  |  |  |  |  |
| 2 | Sector Count ( 0:7 ) |  |  |  |  |  |  |  |
| 3 | Sector Count ( 8:15 ) (For extended command) (2*) |  |  |  |  |  |  |  |
| 4 | LBA Low ( 0:7 ) |  |  |  |  |  |  |  |
| 5 | LBA Low ( 8:15 ) (For extended command) (2*) |  |  |  |  |  |  |  |
| 6 | LBA Mid ( 0:7 ) |  |  |  |  |  |  |  |
| 7 | LBA Mid ( 8:15 ) (For extended command) (2*) |  |  |  |  |  |  |  |
| 8 | LBA High ( 0:7 ) |  |  |  |  |  |  |  |
| 9 | LBA High ( 8:15 ) (For extended command) (2*) |  |  |  |  |  |  |  |
| 10 | Device / Head |  |  |  |  |  |  |  |
| 11 | Feature ( 0:7 ) |  |  |  |  |  |  |  |
| 12 | Feature ( 8:15 ) (For extended command) (2*) |  |  |  |  |  |  |  |
| 13 | Command |  |  |  |  |  |  |  |
| 14 | Device Control (1*) |  |  |  |  |  |  |  |
| 15 | Extent Status (For extended command) (2*) |  |  |  |  |  |  |  |

(1*) The value of the 14th field is not always 0xA0, it is depended on command.

(2*) The extended command field is used in 48 bit LBA mode, these fields can be ignored.

Others fields, please refer to SATA spec.